RS232 Command:

  1.The virtual serial port should be configured as follows.

Bits per second – 115200
Data bits – 8

Parity – None

Stop bits – 1

Flow control – None

2. An explanation of the content transmitted to the display
Tx: [line 1] 20 EF 03 19 00 HH HH II JJ JJ CC CC FF FF FF FF

Tx: [line 2] KK 00 00 00 CC CC CC CC CC CC CC CC CC CC CC Checksum
20 EF – signifies the beginning of the transmission (always 20 EF)

03 – indicates the packet type (typically 03 in this application)
19 00 – indicates the packet length (typically 19 00 in this application)

HH HH – gives the calculated CRC checksum for the transmission

II – gives the operation type

JJ JJ – gives the operation name (least significant byte is on the left)

KK – gives the operation value

All CC, FF, or 00 – not applicable… always CC, FF or 00 as indicated

Checksum – ex: Power On:


0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x00,0x02,0x01,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,Checksum


Step1.0x20+0xEF+0x03+0x19+0x00+0x00+0x00+0x01+0x00+0x02+0x01+0x00+0x00+0x00+0x00+0x00+0x00+0x00+0x00+0x00+0xCC+0xCC+0xCC+0xCC+0xCC+0xCC+0xCC+0xCC+0xCC+0xCC+0xCC=9F3





Step2.0xA00-0x9F3=D

Checksum:0x0D

Step3.Final: 0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x00,0x02,0x01,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0x0D
Application：
//Power On: 0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x00,0x02,0x01,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0D

//Power Off
0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x00,0x02,0x01,0x00,0x00,0x00,0x00,0x00,0x04,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x09
//Auto Adjust

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x02,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0B

//Lock Power

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x01,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0C

//Unlock Power

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x01,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x02,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0B
//Lock OSD

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x01,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x03,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0A
//Unlock OSD

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x01,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x04,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x09
//Factory Reset
0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x03,0x04,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x07
//OSD H-POS

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x03,0x01,0x01,0x00,0x00,0x00,0x00,0x00,Adjust_Scroll_Change(0-100),0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0B

//OSD V-POS

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x03,0x02,0x01,0x00,0x00,0x00,0x00,0x00,Adjust_Scroll_Change(0-100),0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x0A

//OSD TIMEOUT

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x03,0x03,0x01,0x00,0x00,0x00,0x00,0x00,Adjust_Scroll_Change(5-60),0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x04

// SET ENGLISH

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x09
// SET FRANCAIS

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x02,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x08
// SET DEUTSCH

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x03,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x07
// SET ITALIANO

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x04,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x06
//SET ESPANOL

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x05,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x05
//SET 日本語

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x06,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x04
//SET 繁體字

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x07,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x03
//SET 簡體字

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x04,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x08,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x02
//SET 9300K

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x05,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x08
//SET 6500K

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x05,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x02,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x07
//SET 5500K

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x05,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x03,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x06
//SET 7500K

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x05,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x04,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x05
//SET USER

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x05,0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x05,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x04
//ADJUST CONTRAST

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x48,0x02,0x01,0x00,0x00,0x00,0x00,0x00,Adjust_Scroll_Change(0-100),0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0xC5

//ADJUST BRIGHTNESS

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x47,0x02,0x01,0x00,0x00,0x00,0x00,0x00,Adjust_Scroll_Change(0-100),0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0xC6
//ADJUST VOLUME

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x06,0x01,0x01,0x00,0x00,0x00,0x00,0x00,Adjust_Scroll_Change(0-100),0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x08

//MUTE ON

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x06,0x02,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x06
//MUTE OFF

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x06,0x03,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x05
//change to VGA
0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x49,0x10,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0xB5
//change to HDMI

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x49,0x10,0x01,0x00,0x00,0x00,0x00,0x00,0x02,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0xB4
//change to DP

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x49,0x10,0x01,0x00,0x00,0x00,0x00,0x00,0x03,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0xB3
//ROTATION_LANDSCAPE

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x06,0x04,0

x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x04
//ROTATION_PORTRAIT

0x20,0xEF,0x03,0x19,0x00,0x00,0x00,0x01,0x06,0x05,0x01,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x00,0x00,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC,0xCC, 0x03
PS:After RS232 send commands, please empty the array of command bit.
5





14





6





7





14





7





6





5





7





6





5





14





14





7





6





5





6





5





14





7





14





7





6





5





14





7





6





5





6





5





14





7





14





7





6





5





(0->0B; 1->0A; 2->09…..)





14





7





6





5





(0->0A; 1->09; 2->08……)





14





7





6





5





(5->04; 6->03; 7->02……)





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





(0->C5; 1->C4; 2->C3……)





14





7





6





5





(0->C6; 1->C5; 2->C4……)





14





7





6





5





(0->08; 1->07; 2->06……)





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5





14





7





6





5








